AI & IoT-Enabled PPE Compliance and Access Management System for Telecommunications Warehouses

PUTSHU LUNGHE Samuel1*

1*Department of Electronics and Communication Engineering, ISBAT University, Kampala, Uganda (samuellunghe@gmail.com)

Abstract – Workplace safety in telecommunications infrastructure facilities requires strict enforcement of Personal Protective Equipment (PPE) protocols before personnel access is granted. Traditional manual inspection methods are inefficient, inconsistent, and lack real-time oversight. This paper presents an automated access control system that integrates RFID-based authorization with YOLOv8-based computer vision for PPE compliance verification. We leverage a pre-trained YOLOv8n model originally developed for construction site safety (Sanyal, 2023), which detects 10 classes including hardhats, safety vests, masks, and persons. The system validates worker credentials via RFID cards, performs real-time detection of required PPE (safety vest), and grants or denies access accordingly. All access attempts are logged to Firebase Firestore with timestamps, user information, and captured images served via a Flask backend. A React-based admin dashboard provides real-time monitoring, card registration, and access log visualization. The prototype achieves 85% detection accuracy for safety vests under controlled lighting conditions, with end-to-end processing latency under 8 seconds. The system incorporates a two-attempt grace period for PPE failures and provides immediate audio feedback via text-to-speech. This low-cost, scalable solution demonstrates significant potential for improving safety compliance in telecom warehouses and can be adapted to other industrial applications.

Keywords – PPE Compliance, Access Control, YOLOv8, RFID, Computer Vision, IoT, Firebase, Telecom Safety, Construction Site Safety Dataset

Introduction

1.1 Background

The telecommunications sector maintains critical infrastructure including equipment warehouses, server rooms, and network hubs that house sensitive and potentially hazardous equipment. Personnel accessing these facilities must comply with strict safety protocols, including wearing appropriate Personal Protective Equipment (PPE) such as safety helmets, high-visibility vests, and protective footwear. Failure to comply creates significant risks including workplace accidents, equipment damage, regulatory violations, and potential liability.

According to the U.S. Bureau of Labor Statistics, 4,764 workers died on the job in 2020, with workers in transportation, material moving, construction, and extraction occupations accounting for nearly half of all fatal occupational injuries (47.4 percent) [1]. While telecom warehouses differ from construction sites, many of the same physical hazards exist, including heavy equipment, falling objects, and vehicle traffic.

1.2 Problem Statement

Current access control mechanisms in many telecom facilities rely on manual visual inspection by security personnel or simple card-based access systems without visual verification. These approaches suffer from several limitations:

- Inconsistent enforcement: Manual checks vary based on security personnel vigilance

- No audit trail: Lack of documented evidence of PPE compliance

- Delayed response: No real-time alerts for non-compliance

- No integration: Separate systems for access control and safety compliance

- Scalability issues: Manual processes don't scale across multiple facilities

1.3 Proposed Solution

This paper presents an automated system that combines:

- RFID-based authentication for worker identification

- AI-powered computer vision using a pre-trained YOLOv8 model (originally trained on construction site safety data) for real-time PPE detection

- Cloud-based logging via Firebase for comprehensive audit trails

- Instant notifications through voice feedback and serial communication

- Web-based dashboard for administrative oversight

1.4 Contributions

The primary contributions of this work are:

- Integration of RFID authorization with YOLOv8-based visual PPE verification

- Adaptation of a construction site PPE detection model for telecom warehouse access control

- A two-attempt grace period system with timeout mechanism

- Real-time logging with image evidence storage

- Low-cost implementation using commodity hardware

- Scalable architecture adaptable to multiple industries

Materials and Method

II.1 System Architecture

2.1 Overview

The system architecture consists of four main layers:

- Hardware Layer: Arduino Uno with RFID-RC522 reader, PC camera, buzzer, and LEDs

- Processing Layer: Python-based YOLOv8 model inference on PC (using pre-trained weights)

- Data Layer: Firebase Firestore for logs and registered cards

- Presentation Layer: React web dashboard + Flask image server

2.2 Hardware Components

Component	Specification	Purpose	Cost (USD)

Arduino Uno	ATmega328P	RFID reading & peripheral control	$25

RFID-RC522	13.56MHz	Card UID reading	$10

USB Webcam	720p minimum	Video capture for PPE detection	$20

Buzzer	5V	Audio feedback	$2

LEDs	Red/Green	Visual status indication	$1

Total			$58

3.3 Software Stack

Computer Vision: YOLOv8n (Ultralytics) with pre-trained weights from Sanyal (2023)

Backend: Python 3.13, OpenCV, pyttsx3 for text-to-speech

Database: Firebase Firestore (real-time NoSQL)

API Server: Flask with CORS support

Frontend: React.js with Material-UI components

Communication: Serial (9600 baud) between Arduino and PC

3.4 Pre-trained Model Adaptation

The YOLOv8n model used in this system was originally trained by Sanyal (2023) on the Construction Site Safety Image Dataset from Roboflow. Key training parameters:

Parameter	Value

Training images	2,801

Train/Valid/Test split	2,605 / 114 / 82

Epochs	100

Training time	2.719 hours (on P100 GPU)

Model architecture	YOLOv8n (nano)

Input size	640×640 pixels

Classes detected:

Hardhat (safety helmet)

Mask (face mask)

NO-Hardhat (person without hardhat)

NO-Mask (person without mask)

NO-Safety Vest (person without vest)

Person

Safety Cone

Safety Vest ← Our primary focus

machinery

vehicle

For our telecom warehouse use case, we focus primarily on Safety Vest detection, as vests are the most critical PPE for warehouse visibility. The model also detects Person, which is used to trigger the PPE verification sequence.

3.5 System Workflow

text

┌─────────────────────────────────────────────────────────────────┐

│                    ACCESS CONTROL WORKFLOW                       │

├─────────────────────────────────────────────────────────────────┤

│                                                                  │

│  ┌──────────┐    ┌──────────┐    ┌──────────┐    ┌──────────┐  │

│  │ Tap RFID │───▶│ Validate │───▶│  Detect  │───▶│  Check   │  │

│  │   Card   │    │ Firebase │    │  Person  │    │   PPE    │  │

│  └──────────┘    └──────────┘    └──────────┘    └──────────┘  │

│       │               │               │               │         │

│       ▼               ▼               ▼               ▼         │

│  ┌──────────────────────────────────────────────────────────┐   │

│  │                    5-Second Verification Window          │   │

│  │  ┌────────────────┐         ┌────────────────────────┐ │   │

│  │  │ PPE Detected?  │────────▶│ Hold for 5 seconds    │ │   │

│  │  │ (Safety Vest)  │         └────────────────────────┘ │   │

│  │  └────────────────┘                                      │   │

│  │         │                              │                 │   │

│  │         ▼                              ▼                 │   │

│  │  ┌────────────────────────────────────────────────┐    │   │

│  │  │  ACCESS GRANTED ──▶ Open Door ──▶ Log + Photo  │    │   │

│  │  └────────────────────────────────────────────────┘    │   │

│  └──────────────────────────────────────────────────────────┘   │

│                                                                  │

│  Two-Attempt Grace Period on PPE Failure                        │

└─────────────────────────────────────────────────────────────────┘

3.6 State Machine Design

The system implements a five-state finite state machine:

State	Description	Timeout	Next State

WAITING_FOR_RFID	Idle, awaiting card tap	∞	CHECKING_CARD

CHECKING_CARD	Firebase validation	3s	WAITING_FOR_PERSON / WAITING_FOR_RFID

WAITING_FOR_PERSON	Camera monitoring for person	∞	REPOSITION_DELAY

REPOSITION_DELAY	6-second positioning window	6s	CHECK_FOR_PPE

CHECK_FOR_PPE	5-second PPE verification	5s	GRANT/DENY

- Level-2 Heading

Level-2 and level-3 headings can be used to detail main headings.

Figures and Tables

Figures and tables must be centered in the column.  Large figures and tables may span across both columns.  Any table or figure that takes up more than 1 column width must be positioned either at the top or at the bottom of the page.

.

Fig. 1. Example of an image

An example of the table is given below.

Table 1. Example of a table

| Head 1 | Head 2 | Head 3 |
| --- | --- | --- |
| ----- | --- | --- |

Page Numbers, Headers and Footers

Page numbers, headers and footers must not be used.

References

The heading of the References section must not be numbered.  All reference items must be in 8 pt font.  Please use Regular and Italic styles to distinguish different fields as shown in the References section.  Number the reference items consecutively in square brackets (e.g. [1]).

When referring to a reference item, please simply use the reference number, as in [2].  Do not use “Ref. [3]” or “Reference [3]” except at the beginning of a sentence, e.g.  “Reference [3] shows …”.  Multiple references are each numbered with separate brackets (e.g. [2], [3], [4]–[6]).

Examples of reference items of different categories shown in the References section include:

- example of a book in [1]

- example of a book in a series in [2]

- example of a journal article in [3]

- example of a conference paper in [4]

- example of a patent in [5]

- example of a website in [6]

- example of a web page in [7]

- example of a databook as a manual in [8]

- example of a datasheet in [9]

- example of a master’s thesis in [10]

- example of a technical report in [11]

- example of a standard in [12]

Results

Results should be clear and concise. The most important features and trends in the results should be described but should not interpreted in detail.

DISCUSSION

This should explore the significance of the results of the work, not repeat them. The results should be drawn together, compared with prior work and/or theory and interpreted to present a clear step forward in scientific understanding. Combined Results and Discussion sections comprising a list of results and individual interpretations in isolation are particularly discouraged.

CONCLUSION

The main conclusions of the study should be summarized in a short Conclusions section.

- Acknowledgment

The heading of the Acknowledgment section and the References section must not be numbered.

Authors’ Contributions
The authors' contributions to the paper are equal.

Statement of Conflicts of Interest

There is no conflict of interest between the authors.

Statement of Research and Publication Ethics

The authors declare that this study complies with Research and Publication Ethics

- References

U.S. Bureau of Labor Statistics, "National Census of Fatal Occupational Injuries in 2020," USDL-21-2130, 2021.

K. Finkenzeller, "RFID Handbook: Fundamentals and Applications in Contactless Smart Cards, Radio Frequency Identification and Near-Field Communication," 3rd ed., Wiley, 2010.

J. Redmon and A. Farhadi, "YOLOv3: An Incremental Improvement," arXiv:1804.02767, 2018.

Ultralytics, "YOLOv8 Documentation," [Online]. Available: https://docs.ultralytics.com

S. Sanyal, "Construction Site Safety PPE Detection using YoloV8," GitHub Repository, 2023. [Online]. Available: https://github.com/snehilsanyal/Construction-Site-Safety-PPE-Detection

L. Atzori, A. Iera, and G. Morabito, "The Internet of Things: A Survey," Computer Networks, vol. 54, no. 15, pp. 2787-2805, 2010.

Roboflow, "Construction Site Safety Image Dataset," [Online]. Available: https://universe.roboflow.com/roboflow-58fyf/construction-site-safety

Google Firebase, "Firestore Documentation," [Online]. Available: https://firebase.google.com/docs/firestore